- 8 Port Gigabit AI PoE Switch

- 【Introduction】

###### 8 Port Gigabit Intelligent Ethernet PoE Switch, using high-quality high-speed network IC and the most stable PoE chip, the switch incorporates the latest AI functions, equipped with two types port priorities, PoE priority and Network priority, one-key smart. The PoE port meets the 802.3af or 802.3at standard. At the same time, port 1/2 supports HiPoE and the power can reach 60W; The power consumption indicators can help to understand the overall power consumption of the product easily. This series of PoE switches can provide seamless connection for 10/100M Ethernet, and the PoE power supply port can automatically detect and supply power to powered devices that comply with IEEE802.3af or IEEE802.3at standards, and non-PoE devices intelligently detect no power supply, only data is transferred.

###### 【Main Features】

- 8*10/100/1000M rate auto adaptive PoE ports

- Working with IEEE 802.3af/at compliant PDs, expands home and office network

- Flow control mode: full-duplex adopts IEEE 802.3x standard, half-duplex adopts Back pressure standard

- Support Port Auto-flip (Auto MDI/MDIX）

- One-key smart switch, supports VLAN, Default, Extend three working modes

- Support PoE Watchdog Function,Support link monitoring, abnormal self-test and recovery

- Port 1/2 supports HiPoE and the power can reach 60W

- Support sequence port PoE priority, the higher the port priority

- Support port Surge Protection: Common mode 4KV, Differential mode 2KV; ESD:air 8KV，Contact 6KV

【Application Environment】

- Metro Optical Broadband Network: Data network operators such as telecommunications, cable TV, and network system integration,etc.

- Broadband private network: Suitable for financial, government, oil, railway, electric power, public security, transportation, education and other industries

- Multimedia transmission: Integrated transmission of images, voice and data, suitable for remote teaching, conference TV, videophone and other applications

- Real-time monitoring: Simultaneous transmission of real-time control signals, images and data

【Specifications】

| I/O Interface |  |
| --- | --- |
| Power | Input: AC 100-240V, 50/60Hz |
| Ethernet | 8 *10/100/1000M PoE Port |
| Performance |  |
| Switching Capacity | 16Gbps |
| Throughput | 11.9Mpps |
| Packet Buffer | 1Mb |
| MAC Address | 2K |
| Jumbo Frame | 9216bytes |
| Transfer Mode | Store and forward |
| MTBF | 100000 hour |
| Standard |  |
| Network protocol | IEEE802.3 (10Base-T) IEEE802.3u (100Base-TX) IEEE802.3ab (1000Base-TX) IEEE802.3x (Flow control) |
| PoE Protocol | IEEE802.3af （15.4W） IEEE802.3at （30W） IEEE802.3bt （60W） |
| Industry Standard | EMI: FCC Part 15 CISPR (EN55032) class A EMS: EN61000-4-2 (ESD)、 EN61000-4-4 (EFT)、 EN61000-4-5 (Surge) |
| Network Medium | 10Base-T : Cat3、4、5 or above UTP(≤100m) 100Base-TX : Cat5 or above UTP(≤100m) 1000Base-TX : Cat5 or aboveUTP(≤100m) |
| Protection |  |
| Security Certificate | CE、FCC、RoHS |
| Environment |  |
| Working Environment | Working Temperature：-20~50°C Storage Temperature：-40~85°C Working Humidity ：10%~90%，non-condensing Storage Temperature：5%~90%，non-condensing Working Height：Maximum10,000 feet Storage heigh：Maximum 10,000 feet |
| Indication |  |
| LED Indicators | PWR（power supply），Mode（DIP），1-8 Green（Link&Data） |
| PWR | Lighting：Powered Un-Light：No Power |
| Mode | Lighting：VLAN Un-Light：Default Flashing：Extend |
| 1-8 Green | Lighting：Connecting Flashing：Data Transmit Un-Light：Disconnect |
| DIP Switch | VLAN, Default, Extend |
| VLAN | Port Isolation, 1-6 port is isolated each other, but all can communicate with uplink port 7/8, AI enable |
| Default | Normal mode, all port can communicate with each other, The port AI mode is closed. |
| Extend | Extend Mode, 3-6 port support 250 meters transmission on 10Mbps/s ,AI enable |
| Mechanical |  |
| Structure Size | Product Dimension (L*W*H): 200mm*118mm*44mm Package Dimension (L*W*H): 245mm*190mm*60mm N.W：0.6kg G.W：0.9kg |
| Packing Info | Carton MEAS：505*320*400mm Packing Qty：20 units Packing Weight：19KG |
| Power Voltage | Input Voltage ：AC 100-240 V Power supply：52V2.3A |
| Package List | Switch 1 pcs, Power cord 1 pcs, User manual 1 pc, Certification 1 pc |
| Ordering Info |  |
| HR900-AXG-80N | 8-port 10/100/1000M AI PoE Switch |

【Product Picture】

【Application】