6-Port Gigabit PoE Switch

【Introduction】

6-Port Gigabit PoE switch, using high-quality high-speed network IC and the most stable PoE chip,  The PoE port meets the 802.3af or 802.3at standard.  This series of PoE switches can provide seamless connection for 10/100/1000M Ethernet, and the PoE power supply port can automatically detect and supply power to powered devices that comply with IEEE802.3af or IEEE802.3at standards, and non-PoE devices intelligently detect no power supply, only data is transferred.

【Main Features】

- 4*10/100/1000M POE Ports + 1*1000M RJ45 Port+1*1000M SFP Port

- Comply with IEEE 802.3, IEEE 802.3u,IEEE802.3ab,IEEE802.3z standards

- port 1 support BT 40W;

- Ethernet port 10/100/1000M rate auto adaptive

- Flow control mode: full-duplex adopts IEEE 802.3x standard, half-duplex adopts Back pressure standard

- Support Port Auto-flip (Auto MDI/MDIX）

- Panel indicators to monitor working status and help fault analysis

- One-key smart switch, supports VLAN,Default,Extend

【Application Environment】

- Metro Optical Broadband Network: Data network operators such as telecommunications, cable TV, and network system integration,etc.

- Broadband private network: Suitable for financial, government, oil, railway, electric power, public security, transportation, education and other industries

- Multimedia transmission: Integrated transmission of images, voice and data, suitable for remote teaching, conference TV, videophone and other applications

- Real-time monitoring: Simultaneous transmission of real-time control signals, images and data

【Specifications】

| I/O Interface |  |
| --- | --- |
| Power | Input: AC 100-240V, 50/60Hz |
| Ethernet | 4*10/100/1000M PoE Port 1*1000M RJ45 Port 1* Gigabit SFP Uplink Port |
| Performance |  |
| Switching Capacity | 12Gbps |
| Throughput | 8.928Mpps |
| Packet Buffer | 1Mb |
| MAC Address | 2K |
| Jumbo Frame | 9216bytes |
| Transfer Mode | Store and forward |
| MTBF | 100000 hour |
| Standard |  |
| Network protocol | IEEE802.3 (10Base-T) IEEE802.3u (100Base-TX) IEEE802.3ab(1000Base-TX) IEEE802.3z(1000Base-FX) IEEE802.3x (Flow control) |
| PoE Protocol | IEEE802.3af （15.4W） IEEE802.3at （30W） |
| Industry Standard | EMI: FCC Part 15 CISPR (EN55032) class A EMS: EN61000-4-2 (ESD), EN61000-4-5 (Surge) |
| Network Medium | 10Base-T : Cat3、4、5 or above UTP(≤100m) 100Base-TX : Cat5 or above UTP(≤100m) 1000Base-TX : Cat5 or above UTP(≤100m) |
| Optical Media | Multimode fiber : 50/125, 62.5/125, 100/140μm Single mode fiber: 8/125, 8.7/125, 9/125, 10/125μm |
| Protection |  |
| Security Certificate | CE、FCC、RoHS |
| Environment |  |
| Working Environment | Working Temperature：-10~50°C Storage Temperature：-40~70°C Working Humidity ：10%~90%，non-condensing Storage Temperature：5%~90%，non-condensing |
| Indication |  |
| PWR（power supply） | Lighting：Powered Un-Light：No Power |
| Mode（DIP） | Lighting：VLAN Un-Light：Default Flashing：Extend |
| 1-6 Gree（Link&Data） | Lighting：Connecting Flashing：Data Transmit Un-Light：Disconnect |
| DIP Switch | VLAN：Port isolation mode. In this mode, the PoE ports (1-4) of the switch cannot communicate with each other, and can only communicate with the UP-link port. Default : Normal mode, all port can communicate with each other, the transmission distance is within 100 meters Extend：Link extension mode, 3-4 ports PoE power supply and data transmission distance can be extended to 250 meters, the transmission rate becomes 10M |
| Mechanical |  |
| Structure Size | Product Dimension : 200*118*44.5mm Package Dimension : 245*190*60mm N.W：0.63kg G.W：0.93kg |
| Packing Info | Carton MEAS：520*320*400mm Packing Qty：20units Packing Weight：19.6KG |
| Power Voltage | Input Voltage ：AC 100-240 V Power supply：52V1.25A |
| Package List | Switch 1 pcs, Power cord 1 pcs, User manual 1 pc, Certification 1 pc, |
| Ordering Info |  |
| ZX901-AXG-411NS | 6 Port Gigabit PoE Switch ,52V1.25A |

【产品效果】

【应用拓扑】